START-TO-FINISH SETUP

DaVinci 200 to ASTAR Middleware

Follow the screens in this order.

| 1 ToolCommander | 2 Host Interface ON | 3 Port 5000 LISTENING | 4 ASTAR connected | 5 Online Local |
| --- | --- | --- | --- | --- |

| DAVINCI CONTROL PC | ASTAR MIDDLEWARE PC |
| --- | --- |
| ToolCommander Host Interface = Server TCP/IP Port = 5000 | production.yaml hsms_mode = active host = DaVinci IPv4 |

# If all three GEM State fields look gray

Your current GEM State strip.

MEANING: This is not a documented GEM state. Do not call it Offline.

ACTION: Open Components > Host Interface (Global) > Operation.

READ: FabLink Connection, HSMS State, Communications State, and Control State.

EXPECTED: A normal top strip has one colored field: red, yellow, or green.

A gray ToolCommander header monitor labeled Disabled is a separate indicator. It means host communication is disabled or not installed.

# Step 1 - Start the DaVinci

LOCATION: DaVinci cabinet and control screen.

ACTION: If ToolCommander is already visible, do not power-cycle. Go to Step 2.

EXPECTED: ToolCommander is open.

IF WRONG: If the machine is fully off, use the cold-start actions below.

COLD START 1: Turn the Main Power Switch clockwise.

COLD START 2: Wait about 30 seconds.

COLD START 3: Press Main Switch (I).

COLD START 4: Wait for Windows and ToolCommander.

Cold start: Main Power Switch ON, then Main Switch (I). Source: Maintenance Manual, p. 18.

Do not press Init Machine for this setup Initialization can move hardware. It does not open TCP port 5000.

# Step 2 - Log in

LOCATION: ToolCommander > bottom-right User Options... > Log On.

ACTION: Enter an authorized user name and password. The account needs Change Parameters and Control Fab Hostinterface.

EXPECTED: You can save Parameters and operate Offline or Online Local.

IF WRONG: Ask the tool administrator for the missing right.

Log On dialog. Source: Maintenance Manual, p. 16.

Rights required Change Parameters = edit and save. Control Fab Hostinterface = change Offline, Online Local, or Online Remote.

# Step 3 - Open Host Interface (Global)

LOCATION: ToolCommander bottom menu.

ACTION: Press Components. Select Host Interface. Select module Global. Press Operation.

EXPECTED: The window title is Host Interface (Global).

IF WRONG: If Host Interface is absent, stop. Ask the tool administrator to confirm it is installed.

Current screen: Components is on the bottom menu. Host Interface (Global) is open.

The top GEM State strip is only a summary. Use the Operation fields for diagnosis.

# Step 4 - Check the Operation screen

LOCATION: Components > Host Interface (Global) > Operation.

ACTION: Check Operation Mode. It must not be Disabled. Use Production for normal service.

EXPECTED: Production is selected. FabLink Connection is available.

IF WRONG: If Disabled is selected, stop. Ask an authorized tool engineer to select the approved active mode.

Field locations on Operation. Source: Maintenance Manual, p. 16.

Do not copy Maintenance from this manual image The image is a maintenance example. Normal service uses Production. Do not select Online Remote.

# Step 5 - Turn on and save the Host Interface

LOCATION: Components > Host Interface (Global) > Parameters.

ACTION: Check Enable. Select Server. Set TCP/IP Port to 5000. Leave Host Address unchanged. Keep 45/10/5/10/5 and Soft Start 120. Press Save.

EXPECTED: Save becomes gray. After close and reopen, Enable, Server, and 5000 remain.

IF WRONG: If values revert, log in with Change Parameters and save again.

Use these connection values. Press Save, then reopen and verify.

LEAVE ALONE: PPM Send Mode, Loadports, recipe override boxes, and other fields.

SERVER MODE: Host Address is unused. Do not enter the middleware address there.

# Step 6 - Check port 5000 and find the DaVinci IPv4

LOCATION: DaVinci control PC > Windows Start > type PowerShell > open Windows PowerShell.

ACTION: Run the listener command. Then run ipconfig.

EXPECTED: Port 5000 shows Listen. Record the Ethernet IPv4 used by the middleware network.

IF WRONG: If no listener appears, return to Steps 4 and 5. Do not continue to ASTAR.

Get-NetTCPConnection -State Listen -LocalPort 5000

netstat -ano | findstr ":5000"

ipconfig

| Result | Action |
| --- | --- |
| 0.0.0.0:5000 or DaVinci-IPv4:5000 | Continue. |
| 127.0.0.1:5000 only | Stop. Remote middleware cannot reach it. |
| No result | Recheck Enable, Save, Server, port 5000, and Operation Mode. |

Do not use 127.0.0.1 or localhost as the ASTAR host unless ASTAR runs on the DaVinci PC.

# Step 7 - Set the ASTAR machine entry

LOCATION: ASTAR middleware PC > Windows Start > type PowerShell > open PowerShell.

ACTION: Open production.yaml in Notepad. Find endpoint_id TOOL_02. Set the DaVinci IPv4. Save the file.

EXPECTED: TOOL_02 uses port 5000, device ID 0, active mode, and enabled true.

IF WRONG: If the file is elsewhere, use the production.yaml used by the installed service.

notepad C:\SECSGEM_EAP\app\config\production.yaml

- endpoint_id: "TOOL_02"

host: "192.168.254.13"   # replace if Step 6 found another IPv4

port: 5000

secs_device_id: 0

hsms_mode: "active"

enabled: true

request_online: false

DO NOT USE: localhost or 127.0.0.1 for host on a separate middleware PC.

SAVE: File > Save. Close Notepad.

# Step 8 - Test the network connection

LOCATION: ASTAR middleware PC > PowerShell.

ACTION: Stop the installed ASTAR service if it is running. Validate the file. Test TCP from ASTAR to the DaVinci.

EXPECTED: Validation is valid. TcpTestSucceeded is True. test-machine prints tcp-ok.

IF WRONG: Refused: fix the DaVinci listener or port. Timeout: fix the IPv4, route, VLAN, ACL, or firewall.

Get-Service AstarSecsGemEapMiddleware

Stop-Service AstarSecsGemEapMiddleware   # run only if the service exists

cd C:\SECSGEM_EAP\app

python -m eap_middleware validate-config --config config\production.yaml

Test-NetConnection 192.168.254.13 -Port 5000

python -m eap_middleware test-machine --config config\production.yaml --endpoint-id TOOL_02

Stop here unless both TCP tests pass tcp-ok checks the socket only. Continue to the live ASTAR check below.

# Step 9 - Start ASTAR and watch Operation

LOCATION: ASTAR middleware PC PowerShell and DaVinci Host Interface Operation screen.

ACTION: Run ASTAR in the foreground. Keep the PowerShell window open. Watch the four Operation fields.

EXPECTED: ASTAR prints [TOOL_02] Communication established. DaVinci shows FabLink green, HSMS Connected, selected, and Communications Communicating.

IF WRONG: If HSMS does not select, stop other host software and restart ASTAR. If Communications does not reach Communicating, read the ASTAR error and restart once.

python -m eap_middleware run-service --config config\production.yaml

| Operation field | Required result |
| --- | --- |
| FabLink Connection | Green |
| HSMS State | Connected, selected |
| Communications State | Communicating |
| Control State | Equipment Offline is acceptable before Step 10 |

# Step 10 - Select Online Local

LOCATION: DaVinci > Components > Host Interface (Global) > Operation > Mode.

ACTION: Keep ASTAR running. Press Online Local once. Do not press Online Remote.

EXPECTED: Control State becomes Online Local. The top Online Local field becomes yellow.

IF WRONG: If it stays Offline or all gray, confirm ASTAR still shows Communication established. Then repeat Step 9.

| Offline | Online Local | Online Remote |
| --- | --- | --- |

COLOR KEY ONLY. On the machine, one field is colored. The other two are gray.

Example: Offline is red. For this setup, expect Online Local yellow. Source: User Manual, p. 28.

Restart ASTAR once after Online Local Press Ctrl+C in the foreground ASTAR window. Run the Step 9 command again. This reruns status and event setup while the tool is Online Local.

# Step 11 - Confirm the final state

LOCATION: DaVinci Operation screen and ASTAR PowerShell or service log.

ACTION: Check every line below.

EXPECTED: Every line passes.

IF WRONG: Use the fault table. Fix the first failed line only.

- DaVinci Parameters: Enable checked, Server, port 5000, values saved.

- DaVinci Operation Mode: not Disabled.

- DaVinci FabLink Connection: green.

- DaVinci HSMS State: Connected, selected.

- DaVinci Communications State: Communicating.

- DaVinci Control State: Online Local.

- Top GEM State: Online Local yellow.

- ASTAR: [TOOL_02] Communication established.

## If one line fails

| First failed result | Exact next action |
| --- | --- |
| All three top fields gray | Ignore the strip. Read the four Operation fields. |
| Saved values revert | Log in with Change Parameters. Set again. Press Save. |
| Operation Mode is Disabled | Authorized tool engineer selects Production. |
| FabLink is not green | Recheck Enable, Save, and Operation Mode. Stop before ASTAR. |
| No local port 5000 listener | Recheck Server and port 5000. Inspect ToolCommander Messages. |
| Remote TCP is refused | Start the listener or correct the port/firewall rule. |
| Remote TCP times out | Correct the IPv4, route, VLAN, ACL, or firewall. |
| TCP passes; HSMS not selected | Stop another host. Confirm DaVinci Server and ASTAR active. Restart ASTAR. |
| Selected; not Communicating | Restart ASTAR once. Read the first ASTAR protocol error. |
| Communicating; still Offline | Press Online Local while ASTAR is running. |
| S9F1 after connection | Match ASTAR secs_device_id to HostConnection.ini [Connection]. Default is 0. |